AERIS

AI-Enabled Reliability Intelligence & Screening

AI-Driven Anomaly Detection in Component Burn-In & Screening

Smart India Hackathon 2026 — Problem Statement SIH26170

Organization: Indian Space Research Organisation (ISRO)  |  Category: Software  |  Theme: Smart Automation

FULL PROJECT DOCUMENTATION

ML training strategy • dataset engineering • anomaly detection • drift prediction • explainable AI • dashboard • evaluation • deployment

This edition incorporates additional recommended enhancements, shown in green boxes throughout, that extend the original plan.

# Table of Contents

1. Executive Summary

2. Problem Statement and Context

3. Proposed Solution — AERIS

4. Dataset Strategy for ML Training

5. Dataset Acquisition and Governance

6. Target SIH Dataset Design

7. Data Generation and Synthetic Burn-In Engine

8. Data Preprocessing

9. Feature Engineering

10. ML Task A — Dynamic Anomaly Detection

11. ML Task B — Time-Series Drift Prediction

12. Risk Engine

13. Explainable AI

14. Evaluation Strategy

15. Validation and Experimental Design

16. Model Training Pipeline

17. Backend Architecture

18. Database Design

19. Frontend / Dashboard

20. Digital Burn-In Simulator

21. Synthetic Data Generation Formulae

22. Handling Class Imbalance

23. Model Explainability and Auditability

24. Deployment

25. Project Folder Structure

26. Security and Reliability

27. Risks and Mitigations

28. Innovation

29. Expected Outputs

30. Judge Demonstration Flow

31. Team Roles

32. Suggested 36-Hour Hackathon Plan

33. Final Recommended ML Stack

34. Limitations and Scientific Honesty

35. References and Dataset Links

36. Final Project Statement

Appendix A — Example Model Card

Appendix B — Minimum CSV Example

Appendix C — What the Team Should Say About the Dataset

Appendix D — Summary of Enhancements in This Edition

# 1. Executive Summary

SIH26170 asks for an AI-driven approach to detect anomalous behaviour in high-reliability electronic components undergoing Environmental Stress Screening (ESS) / burn-in. Traditional screening relies heavily on static pass/fail limits. The proposed AERIS system adds a predictive layer that learns normal population behaviour and degradation trajectories, detects subtle dynamic outliers, forecasts later-stage drift from early measurements, and explains why a component was flagged.

The central data challenge is that the public SIH problem statement does not currently provide an official component-level burn-in dataset. Therefore, the project does not claim that publicly available ISRO spacecraft telemetry is the SIH training dataset. Instead, AERIS uses a tiered training strategy: (1) real public NASA electronics-aging/prognostics data where raw resources are available, (2) relevant space/telemetry datasets as secondary domain evidence, and (3) a statistically and physics-informed synthetic burn-in dataset specifically shaped to the SIH26170 workflow.

NASA's Open Data Portal currently lists a directly downloadable IGBT accelerated-aging dataset containing aging data from six devices, with electrical and thermal measurements — a particularly relevant source for learning degradation behaviour. NASA also lists electronics prognostics and capacitor-degradation resources, although some catalogue entries provide papers rather than raw data. The documentation therefore distinguishes downloadable raw-data sources from literature/reference sources.

The final SIH model is trained on an SIH-compatible feature space containing early measurements, multi-parameter trajectories, batch effects, temperature/voltage context, anomaly labels and future degradation targets. The system has two core ML tasks: dynamic anomaly detection and early drift prediction. A risk engine combines anomaly severity, predicted drift, uncertainty and failure likelihood, with thresholds selected to heavily penalize false negatives.

RECOMMENDED ENHANCEMENT — Maturity framework Adopt three explicit tiers for every claim made about AERIS: (1) Design concept — an engineering capability proposed in this document, (2) Prototype implementation — working software implemented and tested, (3) Validated engineering claim — a result supported by representative hardware data and engineering review. State which tier each feature occupies. This makes the honesty already present in this document (Section 4.4, Section 34) explicit and easy for judges to check at a glance.

# 2. Problem Statement and Context

Problem ID: SIH26170. Title: AI-Driven Anomaly Detection in Component Burn-In & Screening. Organization: ISRO. Category: Software. Theme: Smart Automation.

High-reliability electronic components may undergo elevated-temperature burn-in/ESS before being approved for use. The objective is to expose early-life weaknesses before integration into critical payloads. A component can remain inside an absolute datasheet limit while its behaviour is nevertheless unusual relative to its lot or while its trajectory indicates progressive degradation.

## 2.1 Core Problem

- Detect population-level outliers even when an individual measurement is below an absolute specification limit.

- Detect abnormal temporal drift before the component reaches a hard failure limit.

- Forecast a later measurement such as Value_168h from early observations such as Value_0h and Value_24h.

- Provide an explanation understandable to a QA/reliability inspector.

- Minimize false negatives, because an undetected defective component can be far more costly than a false alarm.

## 2.2 Illustrative Example

If a lot has a leakage-current mean of approximately 10 µA and a component measures 45 µA while the absolute maximum specification is 50 µA, a static rule may pass it. A dynamic population model can flag it as a severe relative outlier. A time-series model can additionally determine whether its trend is accelerating toward an unsafe region.

# 3. Proposed Solution — AERIS

## 3.1 System Objectives

- Create a realistic training corpus for burn-in anomaly detection without misrepresenting synthetic data as ISRO production data.

- Use real component-aging/prognostics datasets to ground degradation behaviour.

- Detect dynamic multivariate outliers.

- Predict late-stage drift from early-stage measurements.

- Rank components by risk rather than using a single binary pass/fail rule.

- Explain alerts using feature contribution and trajectory evidence.

- Provide a practical dashboard for batch upload, screening, drill-down and report generation.

## 3.2 High-Level Architecture

Data Sources +-- NASA electronics aging / prognostics (primary external training evidence) +-- ISRO/space telemetry (secondary domain validation) +-- Physics/statistics-informed synthetic burn-in (SIH-specific training corpus) ¦ Data Harmonization ¦ Feature Engineering + QC ¦ +------------+-------------+ ¦ ¦ Dynamic Anomaly Detector Drift Predictor (MAD/IQR/Isolation Forest) (XGBoost/gradient model) ¦ ¦ +------------+-------------+ ¦ Risk Engine ¦ Explainability Layer (SHAP + rules) ¦ Web Dashboard / QA Report

# 4. Dataset Strategy for ML Training

## 4.1 Critical Data Principle

External datasets should not simply be concatenated because they represent different devices, units, stress conditions, sampling intervals and failure mechanisms. Instead, they are used in a layered way: raw external data teaches the model family and degradation features; a harmonization layer extracts generic health/degradation concepts; the SIH-specific synthetic corpus supplies the target schema and labels needed for the final task.

## 4.2 Primary Recommended NASA Sources

| Source | What it contains | Role | Caution |
| --- | --- | --- | --- |
| NASA IGBT Accelerated Aging | Aging data from 6 devices; electrical/thermal variables; some high-speed measurements | Primary external component-aging evidence | Small number of devices; must not be treated as a large dataset |
| NASA Towards Prognostics for Electronics Components | IGBT accelerated aging; steady-state voltages/currents, electrical and thermal transients | Feature/prognostics reference and potential training source where raw data is accessible | Catalogue page may expose paper/resource rather than a large raw table |
| NASA IGBT Precursor Parameter Identification | Collector-emitter current, case temperature, gate voltages, collector-emitter voltages; precursor discussion | Feature selection / prognostic methodology | Catalogue entry is primarily a paper |
| NASA Capacitor Degradation / Health Monitoring | Electrical/thermal stress experiments; ESR/capacitance degradation concepts | Secondary degradation modelling evidence | Several catalogue records are papers rather than downloadable raw files |

## 4.3 Why These Are Better Than Generic Spacecraft Telemetry

Electronic component aging data directly captures the kind of phenomenon the SIH problem is concerned with: degradation under stress, changing electrical characteristics and precursor variables. By contrast, spacecraft telemetry can be highly relevant to anomaly detection but may reflect system-level operational events rather than component burn-in physics. Therefore, spacecraft telemetry is best used as secondary validation/reference data rather than as a direct substitute for burn-in measurements.

## 4.4 Public-Data Truthfulness Rule

Never label NASA data as ISRO data. Never label synthetic data as real ISRO production data. In the final presentation, state exactly which datasets are public, which are synthetic, which are derived features, and which labels were generated.

RECOMMENDED ENHANCEMENT — Quantitative synthetic-data validation Before presenting, run a distributional comparison — e.g. a Kolmogorov–Smirnov test or Wasserstein distance — between degradation slopes/curvature in the synthetic corpus and the real NASA IGBT aging data. Include the resulting statistic or plot in the final report. This converts the "physics/statistics-informed" claim from an assertion into demonstrated evidence, which materially strengthens the credibility of the entire training strategy.

# 5. Dataset Acquisition and Governance

## 5.1 Verified NASA Resources

- NASA Open Data: Insulated-Gate Bipolar Transistor (IGBT) Accelerated Aging — the current catalogue lists IGBTAgingData_04022009.zip as a resource and describes six devices under thermal overstress accelerated aging.

- NASA Open Data: Towards Prognostics for Electronics Components — describes IGBT accelerated aging, in-situ steady-state voltages/currents, electrical transients and thermal transients.

- NASA Open Data: Precursor Parameter Identification for IGBT Prognostics — documents monitored current, case temperature, gate voltages and collector-emitter voltages and discusses precursor behaviour.

- NASA Open Data: Integrated Diagnostic/Prognostic Experimental Setup for Capacitor Degradation and Health Monitoring — describes accelerated electrical/thermal stress experiments on electrolytic capacitors.

- NASA Open Data: Prognostic Techniques for Capacitor Degradation and Health Monitoring — describes ESR increase and capacitance decrease under electrical stress.

## 5.2 Source Links

- NASA IGBT Accelerated Aging: data.nasa.gov/dataset/insulated-gate-bipolar-transistor-igbt-accelerated-aging

- NASA Towards Prognostics for Electronics Components: data.nasa.gov/dataset/towards-prognostics-for-electronics-components

- NASA IGBT Precursor Parameter Identification: data.nasa.gov/dataset/precursor-parameter-identification-for-insulated-gate-bipolar-transistor-igbt-prognostics

- NASA Capacitor Diagnostic/Prognostic Setup: data.nasa.gov/dataset/integrated-diagnostic-prognostic-experimental-setup-for-capacitor-degradation-and-health-m

- NASA Capacitor Prognostic Techniques: data.nasa.gov/dataset/prognostic-techniques-for-capacitor-degradation-and-health-monitoring

## 5.3 Data Audit Before Training

- Download each source and record filename, source URL, access date, license, citation and checksum.

- Inspect whether the resource is raw measurements, processed measurements, a PDF, or only a catalogue record.

- Read units and measurement definitions before mapping columns.

- Do not infer a failure label if the source does not provide one; create a derived label only with a documented rule.

- Keep raw files immutable and version processed datasets.

# 6. Target SIH Dataset Design

## 6.1 Recommended Master Schema

| Field | Type | Purpose |
| --- | --- | --- |
| component_id | string | Unique component identifier |
| batch_id | string | Lot/batch identifier for batch effects |
| component_type | category | Device family/type |
| temperature_c | float | Burn-in temperature |
| voltage_v | float | Operating/screening voltage |
| value_0h ... value_168h | float | Measurements at 0/24/48/72/96/120/144/168 hours |
| abs_spec_limit | float | Absolute screening limit |
| lot_mean_0h / lot_std_0h | float | Batch reference / dispersion |
| drift_rate | float | Estimated temporal slope |
| anomaly_type | category | Ground-truth anomaly class |
| anomaly_severity | float | 0–1 or ordinal severity |
| ground_truth_anomaly | 0/1 | Anomaly label |
| ground_truth_failure | 0/1 | Failure/unsafe endpoint label |

## 6.2 Multi-Parameter Extension

For a more realistic project, create separate trajectories for leakage current, Iddq, propagation delay and another electrical parameter. A long-format table can store parameter_name, time_h, value, component_id, batch_id and context. A wide-format modelling table can then be generated for tree models.

RECOMMENDED ENHANCEMENT — Chamber / slot spatial effects Add chamber_id and slot_position fields alongside batch_id. Position within a burn-in oven affects thermal exposure and can create anomalies that are artifacts of test setup rather than the component. Modeling this as a covariate — or at minimum controlling for it when computing the anomaly score — prevents AERIS from over-flagging components purely due to chamber geometry, and gives QA a second axis (equipment vs. component) to investigate an alert.

RECOMMENDED ENHANCEMENT — Lot / wafer genealogy Extend batch_id with an optional wafer_lot_id / fab_batch_id field. Tracing a flagged anomaly back to its wafer lot or fabrication batch gives QA engineers an actionable root-cause path rather than just a per-component alert, and is a low-cost extension of the existing schema.

# 7. Data Generation and Synthetic Burn-In Engine

## 7.1 Why Synthetic Data Is Necessary

The SIH problem requires a burn-in-style time-series structure that the public external datasets do not consistently provide. Synthetic data should therefore reproduce realistic population variation, batch effects, measurement noise, temperature/voltage context and multiple defect trajectories.

## 7.2 Recommended Scale

- Initial prototype: 10,000 components.

- Competition-scale experiment: 50,000–100,000 components if compute and validation time permit.

- 20–100 lots/batches.

- Sampling at 0, 24, 48, 72, 96, 120, 144 and 168 hours.

- Multiple component types and operating conditions.

## 7.3 Normal Trajectory Model

x_i(t) = mu_batch + b_component + beta*t + gamma*temperature_effect + eps_correlated(t) + eps_measurement where: mu_batch = lot-specific baseline b_component = component-level random effect beta = normal drift temperature_effect = operating-condition influence eps_correlated = time-correlated process noise eps_measurement = measurement noise

## 7.4 Anomaly Classes

| Class | Behaviour | Detection signal |
| --- | --- | --- |
| Population outlier | High baseline but stable | Large robust z-score/MAD |
| Gradual drift | Slow monotonic change | Slope / cumulative deviation |
| Accelerating degradation | Slope increases with time | Second derivative / trajectory model |
| Sudden spike | Abrupt measurement jump | Local residual |
| Oscillation | Repeated abnormal variation | Frequency/variance features |
| Thermal runaway proxy | Strong stress-linked growth | Temperature-adjusted slope |
| Hidden latent defect | Inside absolute spec but abnormal vs. lot | Relative population score + forecast |

## 7.5 Label Policy

Use separate labels for anomaly type, anomaly severity and ultimate failure. This prevents the model from collapsing every unusual observation into a single binary class.

# 8. Data Preprocessing

## 8.1 Pipeline

Raw files -> schema validation -> unit normalization -> timestamp/time normalization -> duplicate removal -> missing-value policy -> outlier review -> batch/component grouping -> feature engineering -> train/validation/test split -> model-ready dataset

## 8.2 Missing Values

- Never forward-fill a missing late-stage measurement into a training target.

- For predictor features, use model-appropriate imputation only when the missingness mechanism is understood.

- Add missingness indicators where absence itself may carry information.

- Keep a raw-data quality report.

## 8.3 Leakage Prevention

Never randomly split rows from the same component into train and test. Measurements from the same physical component are correlated. Prefer component-level splits; for stronger validation, split by batch/lot so an entire lot is held out from training.

# 9. Feature Engineering

## 9.1 Early-Value Features

- value_0h, value_24h

- absolute change = value_24h − value_0h

- relative change = (value_24h − value_0h) / |value_0h|

- temperature and voltage

- batch-normalized value

## 9.2 Trend Features

- Linear slope over available points

- Robust Theil–Sen slope

- Maximum local slope

- Curvature / acceleration

- Cumulative absolute change

- Residual from batch trend

- Time-to-threshold estimate

## 9.3 Population Features

- Robust z-score using median and MAD

- Percentile within batch

- Mahalanobis distance for multivariate deviations

- Distance to learned normal cluster

- Isolation Forest score

# 10. ML Task A — Dynamic Anomaly Detection

## 10.1 Recommended Model Stack

Use a hybrid detector instead of one algorithm. A robust statistical detector provides transparent screening; an unsupervised ML detector catches nonlinear/multivariate behaviour.

| Model | Role | Advantage |
| --- | --- | --- |
| Robust Z / MAD | First-pass dynamic outlier | Simple, explainable |
| IQR / percentile rules | Population screening | Easy for QA |
| Isolation Forest | Nonlinear multivariate anomaly detection | Good baseline for mixed features |
| Mahalanobis distance | Correlated parameter anomalies | Useful for multivariate electrical measurements |
| Autoencoder (optional) | Complex nonlinear reconstruction | Use only after strong baseline validation |

## 10.2 Example Dynamic Score

DynamicScore = w1 * RobustPopulationScore + w2 * IsolationForestScore + w3 * TrendDeviationScore + w4 * CrossParameterInconsistency

Calibrate the weights on validation data rather than choosing them arbitrarily.

RECOMMENDED ENHANCEMENT — Equipment-fault vs. component-fault discrimination Add a statistical check that groups anomalies by chamber_id/slot_position (Section 6.2 enhancement) in addition to batch_id. If anomalies cluster by equipment position rather than by lot, surface this as a distinct alert type ("possible equipment fault") rather than a component defect. This addresses a common real-world QA complaint — false alarms caused by a faulty chamber channel, thermocouple, or fixture rather than the component itself — and requires no new model, only a grouped aggregation on existing fields.

# 11. ML Task B — Time-Series Drift Prediction

## 11.1 Target

Predict Value_168h from early observations, with the SIH problem specifically motivating early measurements such as Value_0h and Value_24h.

## 11.2 Baseline Models

| Model | Use |
| --- | --- |
| Linear regression | Transparent baseline |
| Random Forest | Nonlinear baseline |
| XGBoost / gradient boosting | Recommended primary tabular model |
| LSTM/GRU | Optional if enough dense sequential data exists |
| Temporal Transformer | Not recommended for the first prototype |

## 11.3 Recommended First Model

Use XGBoost or another gradient-boosting regressor for the first robust prototype. It works well with heterogeneous engineered features, needs less data than deep sequence models, is easier to explain with SHAP and is fast enough for hackathon iteration.

## 11.4 Output

Inputs: value_0h, value_24h, delta_0_24, relative_delta, temperature, voltage, batch statistics, early trend features Output: predicted_value_168h prediction_interval / uncertainty proxy predicted_drift_rate

RECOMMENDED ENHANCEMENT — Concrete uncertainty quantification method Replace the generic "uncertainty proxy" with one specific, implementable method: either (a) quantile regression — train XGBoost at the 10th/50th/90th percentiles for a fast, tree-native interval — or (b) conformal prediction wrapped around the point predictor for a statistically calibrated interval with a coverage guarantee. Feed this interval directly into the risk engine's uncertainty term (Section 12).

# 12. Risk Engine

## 12.1 Risk Formulation

Risk = wA * anomaly_score + wD * predicted_drift_risk + wF * failure_probability + wU * uncertainty_penalty

Weights should be selected using validation data and a cost matrix. Because false negatives are more dangerous, the decision threshold should be selected against a high false-negative cost.

## 12.2 Recommended Risk Levels

| Risk | Meaning | Action |
| --- | --- | --- |
| Low | Normal trajectory | Pass / monitor |
| Medium | Unusual but uncertain | QA review |
| High | Strong anomaly or drift | Hold component |
| Critical | High failure likelihood / severe trajectory | Reject / engineering investigation |

RECOMMENDED ENHANCEMENT — Weight calibration procedure Run a grid search or Bayesian optimization over (wA, wD, wF, wU) against the TotalCost objective defined in Section 14.3, and present the resulting TotalCost-vs-weight curve. This turns the risk formula from a symbolic expression into a calibrated, defensible scoring function and pre-empts the question "how did you choose these weights?"

# 13. Explainable AI

## 13.1 Inspector-Facing Explanation

AERIS should not merely output 'ANOMALY = 1'. It should say what changed, relative to which reference, and why the prediction matters.

COMPONENT: C-004821 STATUS: HIGH RISK Why flagged: 1. Leakage current is at the 99.2 percentile of its batch. 2. 0h->24h change is 4.8x the normal median drift. 3. Model predicts Value_168h above the learned safe trajectory. 4. Temperature-adjusted residual is unusually high. Recommended action: HOLD FOR QA REVIEW

## 13.2 SHAP

For the final XGBoost model, use SHAP to identify the features contributing most to the predicted 168h value or failure-risk score. Keep the UI explanation concise and map model features back to engineering concepts.

# 14. Evaluation Strategy

## 14.1 Anomaly Detection Metrics

- Recall / sensitivity — primary safety metric.

- False-negative rate — primary risk metric.

- Precision — controls inspection workload.

- F1 score — balanced summary.

- PR-AUC — preferred when anomalies are rare.

- ROC-AUC — secondary metric.

- False alarms per 1,000 components — practical QA metric.

## 14.2 Drift Prediction Metrics

- MAE — primary error measure.

- RMSE — penalizes large misses.

- MAPE only when target values are safely away from zero.

- Prediction interval coverage if uncertainty intervals are implemented.

## 14.3 Safety-Weighted Objective

TotalCost = C_FN * FN + C_FP * FP + C_UNCERTAIN * uncertain_cases where C_FN >> C_FP

Do not optimize only for accuracy. With rare defects, a model can achieve high accuracy by predicting everything as normal.

RECOMMENDED ENHANCEMENT — Baseline comparison against static pass/fail screening The project's central thesis is that AERIS catches latent defects that fixed specification limits miss. Add one evaluation table comparing recall on the "hidden latent defect" class, and false alarms per 1,000 components, for (a) static pass/fail against the absolute spec limit alone, versus (b) AERIS's full pipeline. Even directionally correct numbers from the synthetic test set convert the project's headline claim into demonstrated evidence rather than assertion, and this is likely the single most persuasive exhibit in the final presentation.

| Method | Recall on hidden latent defects | False alarms / 1,000 components |
| --- | --- | --- |
| Static pass/fail (absolute spec limit only) | [to be measured] | [to be measured] |
| AERIS (lot-relative + Isolation Forest + drift prediction) | [to be measured] | [to be measured] |

# 15. Validation and Experimental Design

## 15.1 Split Design

| Split | Recommended rule |
| --- | --- |
| Train | 70% of components/batches |
| Validation | 15% of unseen components/batches |
| Test | 15% of completely held-out components/batches |

For the strongest experiment, perform a batch-held-out test where entire lots are absent from training. This tests generalization to a new production lot.

## 15.2 Stress Tests

- Different batch means and variances.

- Higher measurement noise.

- Missing 24h measurement.

- Temperature shifts.

- New component type.

- Rare anomaly rates.

- Anomalies inside the absolute specification limit.

- Drift acceleration not seen in training.

# 16. Model Training Pipeline

- Acquire external data

- Register source metadata

- Inspect raw files

- Parse measurements

- Normalize units

- Extract generic degradation features

- Generate SIH-compatible synthetic burn-in corpus

- Add labels and ground truth

- Split by component/batch

- Train statistical anomaly baseline

- Train Isolation Forest

- Train XGBoost drift predictor

- Calibrate risk threshold

- Evaluate on held-out test

- Generate SHAP explanations

- Freeze model + scaler + feature schema

- Deploy API

# 17. Backend Architecture

React / TypeScript Dashboard ¦ v FastAPI REST API ¦ +-------+---------+ v v v Screen Predict Explain ¦ ¦ ¦ +-------+---------+ ¦ ML Service +-------+---------+ v v Anomaly Model Drift Model ¦ ¦ +--------+--------+ v Risk Engine ¦ v PostgreSQL

## 17.1 Suggested API Endpoints

| Endpoint | Purpose |
| --- | --- |
| POST /screen/batch | Upload batch measurements |
| POST /predict/drift | Predict late-stage value |
| POST /detect/anomaly | Compute anomaly scores |
| POST /risk/score | Compute risk classification |
| GET /components/{id} | Retrieve component history |
| GET /components/{id}/explanation | Retrieve explanation |
| GET /batches/{id}/summary | Batch-level screening summary |
| GET /models/current | Model version metadata |

# 18. Database Design

components component_id (PK), batch_id, component_type, status measurements component_id, time_h, parameter_name, value, temperature_c, voltage_v predictions component_id, predicted_168h, drift_rate, anomaly_score, risk_level, model_version explanations component_id, feature, contribution, reason_text batches batch_id, component_type, screening_date, operator, summary

# 19. Frontend / Dashboard

## 19.1 Main Dashboard

- Total components screened

- Normal / review / high / critical counts

- False-negative-sensitive alert indicator

- Batch health score

- Anomaly trend

- Top suspicious components

- Model version

## 19.2 Component Drill-Down

- Component metadata

- All parameter trajectories

- Batch mean and confidence band

- Component deviation from batch

- Predicted 168h value

- Risk score

- SHAP/feature contribution explanation

- Recommended QA action

# 20. Digital Burn-In Simulator

A major demo feature should be a simulator that generates a batch in real time. This lets judges see the AI react to a component whose trajectory starts normal and then becomes abnormal.

Simulator: create batch generate normal trajectories inject selected defect stream measurements update anomaly score forecast 168h raise alert show explanation

## 20.1 Demonstration Scenario

- Create 500 components.

- Set 5 hidden latent defects and several mild drift cases.

- Show the static specification line.

- Show that some components remain below the hard limit.

- Let AERIS identify components that are abnormal relative to the lot.

- Display predicted 168h trajectory.

- Open the explanation panel and show the evidence.

# 21. Synthetic Data Generation Formulae

## 21.1 Gradual Drift

x(t) = x0 + beta*t + eps(t)

## 21.2 Accelerating Degradation

x(t) = x0 + beta*t + alpha*t^2 + eps(t), alpha > 0

## 21.3 Sudden Shift

x(t) = x_normal(t) + Delta * I(t >= t_event)

## 21.4 Temperature Effect

x_adjusted(t) = x(t) + k_T * (T(t) - T_ref)

## 21.5 Batch Effect

x_batch = mu_global + delta_batch + delta_component

The coefficients and distributions should be estimated from real external component-aging data when possible, then validated by engineering plausibility checks.

# 22. Handling Class Imbalance

- Do not create an unrealistic 50/50 normal/anomaly dataset.

- Use rare anomalies with realistic prevalence.

- Use class-weighted loss or threshold tuning.

- Evaluate precision-recall curves.

- Use targeted anomaly generation to ensure each failure mechanism is represented.

- Keep the final test prevalence fixed and hidden from model tuning.

# 23. Model Explainability and Auditability

- Store model version with every prediction.

- Store feature schema version.

- Store preprocessing/scaler version.

- Store prediction timestamp.

- Store explanation features and contributions.

- Keep an audit record of threshold changes.

- Never silently retrain the production model.

RECOMMENDED ENHANCEMENT — Post-deployment model drift monitoring Add a scheduled comparison of live prediction-error distributions against the validation baseline, with a defined trigger (e.g. a statistically significant shift in residuals or anomaly-rate) for flagging when a retraining review is warranted. This closes the loop on the existing versioning discipline above: it is not enough to version models — the system should also detect when a version is quietly becoming stale.

# 24. Deployment

## 24.1 Recommended Stack

| Layer | Technology |
| --- | --- |
| Frontend | React + TypeScript + Tailwind CSS |
| Charts | ECharts or Plotly |
| Backend | Python + FastAPI |
| ML | scikit-learn + XGBoost + SHAP |
| Data | Pandas + NumPy |
| Database | PostgreSQL |
| Deployment | Docker |
| Realtime (optional) | WebSocket / MQTT |
| Testing | pytest + API tests |

## 24.2 Container Layout

docker-compose +-- frontend +-- api +-- ml-service +-- postgres

RECOMMENDED ENHANCEMENT — Explicit offline / air-gapped deployment State as an explicit design requirement that the full stack (Docker + PostgreSQL + FastAPI + local model artefacts) runs without any internet dependency, since ISRO burn-in facilities plausibly require on-premise, air-gapped operation. Confirm this is true of the current stack and present it as a stated capability rather than an implicit property.

# 25. Project Folder Structure

aeris/ ¦-- data/ ¦ ¦-- raw/ ¦ ¦ ¦-- nasa/ ¦ ¦ +-- isro/ ¦ ¦-- processed/ ¦ +-- synthetic/ ¦-- notebooks/ ¦ ¦-- 01_data_audit.ipynb ¦ ¦-- 02_feature_engineering.ipynb ¦ ¦-- 03_anomaly_model.ipynb ¦ +-- 04_drift_model.ipynb ¦-- src/ ¦ ¦-- data/ ¦ ¦-- features/ ¦ ¦-- models/ ¦ ¦-- risk/ ¦ ¦-- explainability/ ¦ +-- api/ ¦-- frontend/ ¦-- tests/ ¦-- models/ ¦-- configs/ ¦-- Dockerfile ¦-- docker-compose.yml +-- README.md

# 26. Security and Reliability

- Validate uploaded CSV schema and file size.

- Reject unexpected columns unless explicitly mapped.

- Sanitize identifiers and filenames.

- Log prediction failures.

- Use role-based access if deployed beyond a demo.

- Keep raw evidence immutable.

- Back up database and model artefacts.

- Never overwrite a deployed model without versioning.

# 27. Risks and Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Synthetic data too simplistic | High | Use real component-aging distributions and multiple anomaly mechanisms |
| Data leakage | Very high | Split by component and preferably batch |
| External datasets not raw | Medium | Use them as reference and only train on actual downloadable measurements |
| Domain mismatch | High | Use external data to learn degradation features, then SIH-specific synthetic data for final target |
| Too many false alarms | High | Calibrate thresholds and evaluate false alarms per batch |
| False negatives | Critical | Use recall/PR-AUC and high FN cost |
| Model not explainable | High | SHAP + rule-based engineering explanations |
| Overfitting to synthetic generator | High | Hold out generator parameters and anomaly mechanisms for test |

# 28. Innovation

- Dynamic population-relative screening instead of only fixed specification limits.

- Early prediction of late-stage burn-in behaviour.

- Hybrid statistical + ML anomaly detection.

- Physics/statistics-informed synthetic training corpus when official data is unavailable.

- False-negative-aware risk calibration.

- Explainable QA-oriented decision support.

- Batch-aware modelling that distinguishes manufacturing variation from true component defects.

- Optional transfer/validation using real aerospace electronics-aging data.

RECOMMENDED ENHANCEMENT — Additional differentiators Equipment-fault vs. component-fault discrimination (Section 10), chamber/slot spatial-effect modeling (Section 6), lot/wafer genealogy tracing (Section 6), and an explicit offline/air-gapped deployment claim (Section 24) are not yet standard in comparable SIH26170 submissions and would extend this innovation list meaningfully.

# 29. Expected Outputs

| Output | Description |
| --- | --- |
| ML anomaly model | Dynamic anomaly score and class |
| Drift model | Predicted Value_168h and drift rate |
| Risk engine | Low/Medium/High/Critical classification |
| Explainability | Top reasons for each alert |
| Dataset pipeline | Raw -> processed -> synthetic -> model-ready |
| Dashboard | Batch and component screening interface |
| API | Programmatic screening endpoints |
| Report generator | QA-ready screening report |
| Model card | Training data, limitations, metrics and version |

# 30. Judge Demonstration Flow

- Open AERIS dashboard.

- Upload a burn-in batch CSV.

- Show the batch summary and distribution.

- Show that most components lie in the normal population.

- Highlight a component that is still inside the absolute specification.

- Show its abnormal batch-relative score.

- Show 0h and 24h inputs and the predicted 168h value.

- Show risk level and confidence/uncertainty.

- Open SHAP explanation.

- Generate a QA report.

## 30.1 Key One-Minute Pitch

AERIS does not replace conventional burn-in screening. It adds an intelligence layer on top of it. Instead of asking only whether a component has crossed a fixed limit, AERIS asks whether the component is behaving abnormally for its population and whether its early trajectory predicts an unsafe future state.

# 31. Team Roles

| Role | Responsibilities |
| --- | --- |
| ML engineer | Data pipeline, anomaly model, drift model, evaluation |
| Data engineer | Dataset acquisition, cleaning, synthetic generator |
| Backend engineer | FastAPI, database, model serving |
| Frontend engineer | Dashboard, charts, drill-down |
| Domain/reliability lead | Burn-in assumptions, labels, validation and presentation |

# 32. Suggested 36-Hour Hackathon Plan

| Time | Deliverable |
| --- | --- |
| 0–3h | Repo, architecture, data schema |
| 3–8h | Dataset audit + synthetic generator |
| 8–13h | Feature engineering + anomaly baseline |
| 13–18h | XGBoost drift model + evaluation |
| 18–23h | Risk engine + SHAP |
| 23–29h | FastAPI + database |
| 29–34h | Dashboard + simulator |
| 34–36h | Testing, report, PPT and demo rehearsal |

RECOMMENDED ENHANCEMENT — Schedule risk mitigation Define an explicit MVP cutline — the minimum feature set that must work by hour 30, with everything past that treated as stretch — and pre-record a backup demo video during a mid-point rehearsal so a live-demo failure does not cost the presentation. The dashboard/simulator block (29–34h) and the testing/report/PPT block (34–36h) are the highest-risk points for schedule slippage.

# 33. Final Recommended ML Stack

For the hackathon, keep the first production-quality version deliberately simple and defensible: Robust MAD/IQR + Isolation Forest for anomaly detection; XGBoost regression for 168h drift prediction; SHAP for explainability; a calibrated cost-sensitive threshold for final risk classification. Deep learning should be an optional extension, not the foundation.

## 33.1 Why Not Start With LSTM?

The SIH target can be represented with a small number of fixed screening checkpoints. Tree-based models are easier to train, debug and explain under hackathon time constraints. An LSTM/GRU becomes useful if the project later receives dense, high-frequency time-series data from actual burn-in equipment.

# 34. Limitations and Scientific Honesty

- The public SIH26170 problem statement does not currently expose an official component-level burn-in training dataset.

- Public NASA datasets vary in device type and measurement protocol.

- Some NASA catalogue entries are papers rather than downloadable raw data; only resources verified as actual data should be used as training files.

- Synthetic data cannot prove performance on real ISRO production hardware.

- The final system should be presented as a decision-support/prototype system until validated on representative real screening data.

RECOMMENDED ENHANCEMENT — Working-prototype evidence gap The single highest-priority item before submission: this documentation describes the intended system in full, but a working, runnable slice — synthetic generator, two trained models, a minimal API, and one dashboard view — needs to exist and be demoable. A thorough plan with no visible running code will read as "planned," not "built," during judging.

# 35. References and Dataset Links

- SIH26170 current problem listing — sih2026-ps-viewer.vercel.app/ps/SIH26170

- NASA — IGBT Accelerated Aging — data.nasa.gov/dataset/insulated-gate-bipolar-transistor-igbt-accelerated-aging

- NASA — Towards Prognostics for Electronics Components — data.nasa.gov/dataset/towards-prognostics-for-electronics-components

- NASA — IGBT Precursor Parameter Identification — data.nasa.gov/dataset/precursor-parameter-identification-for-insulated-gate-bipolar-transistor-igbt-prognostics

- NASA — Integrated Diagnostic/Prognostic Capacitor Setup — data.nasa.gov/dataset/integrated-diagnostic-prognostic-experimental-setup-for-capacitor-degradation-and-health-m

- NASA — Prognostic Techniques for Capacitor Degradation — data.nasa.gov/dataset/prognostic-techniques-for-capacitor-degradation-and-health-monitoring

- NASA — Physics-based Capacitor Degradation — data.nasa.gov/dataset/physics-based-degradation-modeling-and-prognostics-of-electrolytic-capacitors-under-electr

# 36. Final Project Statement

AERIS is an explainable AI-based burn-in intelligence platform for high-reliability electronic components. It combines dynamic population-aware anomaly detection with early degradation forecasting so that components showing suspicious behaviour can be identified before they cross conventional screening limits. The training strategy deliberately separates public evidence from SIH-specific synthetic data: real aerospace electronics-aging datasets provide degradation grounding, while a controlled synthetic burn-in corpus supplies the 0h/24h/96h/168h structure and ground truth needed by SIH26170. This produces a transparent, reproducible and technically defensible prototype without falsely claiming access to confidential ISRO production data.

# Appendix A — Example Model Card

| Item | Value |
| --- | --- |
| Model name | AERIS Anomaly + Drift v1 |
| Primary task | Dynamic anomaly detection + 168h drift prediction |
| Training data | Public electronics-aging data + SIH-specific synthetic burn-in |
| Primary models | Isolation Forest + XGBoost |
| Explainability | SHAP + engineering rules |
| Primary safety metric | Recall / false-negative rate |
| Primary regression metric | MAE |
| Split policy | Component/batch-held-out |
| Known limitation | Synthetic data is not production ISRO data |

# Appendix B — Minimum CSV Example

component_id,batch_id,temperature_c,voltage_v,value_0h,value_24h,value_96h,value_168h,drift_rate,anomaly_type,ground_truth_anomaly C00001,B001,125,5.0,10.2,10.3,10.5,10.7,0.003,normal,0 C00002,B001,125,5.0,10.1,10.9,12.2,14.8,0.028,accelerating_drift,1 C00003,B001,125,5.0,10.0,10.1,10.2,10.3,0.002,normal,0 C00004,B001,125,5.0,10.4,11.5,11.7,11.8,0.010,gradual_drift,1

# Appendix C — What the Team Should Say About the Dataset

Recommended wording: "No public SIH26170 production burn-in dataset is currently linked in the problem statement. We therefore built a reproducible SIH-compatible synthetic burn-in corpus and grounded its degradation behaviour using public aerospace electronics-aging/prognostics resources. We clearly separate source data, derived features and synthetic labels."

Avoid saying: "This is ISRO's training dataset," unless ISRO officially provides such data to your team.

# Appendix D — Summary of Enhancements in This Edition

The following enhancements have been integrated throughout this document (shown in green boxes at their relevant sections) beyond the original plan:

- Maturity framework for claims (Section 1)

- Quantitative synthetic-data validation against real NASA data (Section 4)

- Chamber/slot spatial-effect modeling (Section 6)

- Lot/wafer genealogy tracing (Section 6)

- Equipment-fault vs. component-fault discrimination (Section 10)

- Concrete uncertainty quantification method — quantile regression or conformal prediction (Section 11)

- Risk-engine weight calibration procedure (Section 12)

- Baseline comparison against static pass/fail screening (Section 14)

- Post-deployment model drift monitoring (Section 23)

- Explicit offline / air-gapped deployment requirement (Section 24)

- Schedule risk mitigation for the 36-hour plan (Section 32)

- Working-prototype evidence gap flagged as top priority (Section 34)